- Combinational circuit: no memory elements  (inputs) ⇒ (outputs)

Types of Sequential Circuits

- Synchronous seq. ckt: – Its behavior can be defined from the knowledge of its signals at discrete instants of time. – Storage elements: e.g., flip-flops

- Asynchronous seq ckt: – Its behavior depends upon the input signals at any instant of time and the order in which the inputs change. – Storage elements: e.g., latches, feedback paths – Disadvantage: may be unstable & difficult to design

Synchronous Sequential Circuits

- Clocked seq ckts is the  most commonly used sync seq ckts is syn seq ckts .

- They use clock pulses in the inputs of storage elements

- It has a master-clock generator to generate a periodic train of clock pulses ¾

- The clock pulses are distributed throughout the system. ¾

- Storage elements are affected only w/ the arrival of each pulse.

- Adv.: seldom manifest instability problems

- Storage elements: flip-flops

- Flip-flop: a binary cell capable of storing one bit of information

- The state of a flip-flop can change only during a clock pulse transition.

⇒  The transition from one state to the next occurs only at predetermined time intervals dictated by the clock pulses.

(Sync) Sequential Circuit Analysis

- Synchronous seq ckt: — includes f-fs, the clock inputs driven directly or indirectly by a clock signal and the direct sets and resets are unused during the normal functioning of the ckt

- The behavior of a seq ckt is determined from the inputs, outputs, and present state of the ckt. 

- Analysis of a seq ckt: — obtain a suitable description that demonstrates the time sequence of inputs, outputs, and states.

Analysis w/ D Flip-Flops

Example 1.

Example 2.

Analysis JK Flip-Flops

(Analysis of J/k Flip flops and T flip-flops is also done applying the same steps.)

(Sync) Sequential Circuit Design

The following steps /procedural steps are undertaken for the designing of the synchronous sequential Circuit:

- Specification: Writing a specification for the ckt.

- Formulation: Obtain either a state diagram or state table from the statement of the problem.

* State reduction: Reduce the # of states if necessary.

- State assignment: Assign binary codes to the states and obtain the binary-coded state table.

- Flip-flop input equation determination: Select the flip-flop type or types. Derive the flip-flop input equations from the next-state entries in the encoded state table.

- Output equation determination: Derive output equations from the output entries in the state table.

- Optimization: Optimize the flip-flop input equations and output equations.

- Technology mapping: Draw a logic diagram of the ckt using flip-flops, ANDs, ORs, and inverters. Transform the logic diagram to a new diagram using the available flip-flop and gate technology.

- Verification: Verify the correctness of the final design.